HealthSync Personal Health Monitoring System Complete Project Documentation & Technical README

| Version: 1.0.0 | Status: Production Ready | Risk Engine: ML-Powered |
| --- | --- | --- |

# 1. Project Overview

HealthSync is a web-based, real-time health monitoring platform designed for old age homes, hospitals, and care facilities. It enables supervisors to track the health metrics of multiple individuals simultaneously through a single centralized dashboard — eliminating the need for manual rounds and reducing critical-event response times.

The system was originally conceived by an old age home owner who needed a digital solution to monitor elderly residents' daily vitals, activity levels, and sleep patterns without requiring physical presence. HealthSync fulfills this vision through an end-to-end pipeline: from wearable fitness tracker to cloud database to web dashboard.

Core Objectives

- Monitor multiple patients/residents from a single supervisor screen

- Track daily, weekly, and monthly health metrics: BP, Steps, Exercise time, Sleep schedule, Heart Rate, Calories, Distance

- Automatically detect health deterioration using ML-based pattern analysis

- Send real-time risk notifications (Normal / Medium / High Risk)

- Support multiple supervisors with separate login and patient assignment

- Store all data persistently in a cloud database (Firebase)

# 2. System Architecture

## 2.1 High-Level Architecture

HealthSync consists of three major layers:

- Data Collection Layer — Wearable device + companion app + Health Connect bridge

- Backend Layer — Firebase Realtime Database (24/7 live sync)

- Frontend Layer — React.js web dashboard with ML inference engine

## 2.2 Data Flow Pipeline

The complete data pipeline from wearable device to dashboard is:

| ① Watch Amazefit Pace | → | ② Zepp App Official Sync App | → | ③ Health Connect Google's New API | → | ④ Sync App Our Android App | → | ⑤ Firebase Realtime DB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Final Notation: Watch → Zepp App → Health Connect (SDK) → Health Connect Sync (our custom Android app) → Firebase Realtime Database → HealthSync Web Dashboard

# 3. Technology Stack

| Layer | Technology | Purpose |
| --- | --- | --- |
| Frontend | React.js | Web dashboard UI |
| Frontend | Chart.js / Recharts | Health metric visualizations & histograms |
| ML Engine | K-Means Clustering | Current day health analysis & risk classification |
| ML Engine | Trend / Deterioration Model | Long-range pattern detection for declining health |
| Backend | Firebase Realtime DB | 24/7 live cloud data storage & sync |
| Mobile Bridge | Health Connect SDK (Android) | Reads synced data from Zepp App |
| Mobile Bridge | Custom Android App (Health Connect Sync) | Pulls Health Connect data & pushes to Firebase |
| Wearable | Amazefit Pace (Xiaomi) | Fitness tracking device worn by patient |
| Wearable App | Zepp (Official) | Syncs watch data to Health Connect |
| Auth | Firebase Auth | Supervisor and doctor login management |

# 4. Key Features

## 4.1 Multi-Patient Dashboard

Supervisors log in and see all assigned patients on one screen. Each patient card shows live values for:

- Heart Rate (BPM)

- Steps (daily count)

- Calories Burned (kcal)

- Distance Covered (km)

- Sleep Duration (hours)

- Workout/Exercise Time (minutes)

## 4.2 Risk Level Indicator

Every patient profile displays a color-coded Risk Level badge — Normal, Medium Risk, or High Risk — automatically computed by the trained ML model based on the Fitbit fitness tracker dataset.

## 4.3 ML Analysis Modes

Two separate analysis modes are available via dedicated buttons on the dashboard:

| Current Health Analysis Analyzes today's data using K-Means ML clustering. Determines which health cluster the patient falls into based on current readings and assigns a real-time risk score. | Detect Health Deterioration Analyzes past historical trends to detect declining health patterns over time. Identifies gradual metric drops (steps, sleep, workout) that indicate long-range deterioration. |
| --- | --- |

## 4.4 Health History & Histograms

Every data point recorded by the patient is saved to Firebase with a timestamp. The system automatically builds a historical bar chart (normalized scale) combining Heart Rate, Steps, Calories, and Distance across the full timeline — enabling supervisors and doctors to visually review trends week-over-week or month-over-month.

## 4.5 Multi-Role Authentication

- Supervisor Login: Can view all assigned patients, manage notifications

- Doctor Login: Has a separate login and can only view their own assigned patients

- Firebase Auth: Manages all user credentials securely

## 4.6 Notifications

Real-time notifications are triggered on the screen whenever any health factor crosses a risk threshold — particularly when values drop significantly or if deterioration is detected. Notification types include:

- Normal — All metrics within expected range

- Medium Risk — One or more metrics trending downward

- High Risk — Critical drop or accelerated deterioration detected

# 5. Key Technical Challenge: Wearable-to-Web Data Pipeline

## 5.1 Problem Statement

The biggest technical challenge in this project was determining which smartwatch to use and, more critically, how to reliably transfer data from that watch to the web dashboard. The client's original requirement was straightforward: use a website and bring data from the watch to the site using the Google Fit API.

## 5.2 Google Fit API Deprecation

Critical Issue: In 2024, Google officially deprecated the Google Fit API. This API was the original bridge planned for reading watch data. After deprecation, the API could no longer be used for new integrations, completely blocking our initially planned data flow.

## 5.3 Device Selection Challenge

The client proposed the Xiaomi Amazefit Pace as the target wearable device. While this device has a good set of health sensors, it does not natively expose an open API for third-party applications — it requires syncing through the official Zepp companion application.

## 5.4 Solution: Health Connect Bridge

After researching alternatives to Google Fit, the team identified Google Health Connect as the official successor API. Health Connect is the new standard for health data interoperability on Android.

The solution was architected as follows:

Step 1 — Zepp App Setup

The official Zepp app was installed and configured to sync with the Amazefit Pace. This sync pushes all health metrics (heart rate, steps, sleep, etc.) from the watch into the Zepp ecosystem.

Step 2 — Zepp → Health Connect Sync

Zepp was configured to sync its data into Google Health Connect — Google's official replacement for Google Fit. Health Connect acts as a secure, standardized health data repository on the Android device.

Step 3 — Custom Android App (Health Connect Sync)

A custom Android application was developed by the team — named "Health Connect Sync". This app uses the official Health Connect SDK to read health data from the Health Connect repository on the device.

Step 4 — Firebase Upload

The Health Connect Sync app then pushes all retrieved health data to the Firebase Realtime Database. From Firebase, the HealthSync web dashboard reads and displays the data in real-time, 24/7.

## 5.5 Final Data Pipeline Summary

| Watch | Amazefit Pace (Xiaomi) — worn by patient, collects biometrics |
| --- | --- |
| Zepp App | Official companion app, syncs watch data to Health Connect |
| Health Connect | Google's new health data API (replaced Google Fit 2024) |
| Health Connect Sync | Our custom Android app — reads HC data via SDK |
| Firebase | Realtime Database — stores all patient health data 24/7 |
| Web Dashboard | HealthSync React app — reads Firebase and renders dashboard |

# 6. Machine Learning Model

## 6.1 Training Dataset

The ML models were trained using the publicly available Fitbit Fitness Tracker dataset. This dataset contains multi-day health metrics from real users, including step counts, heart rate values, calories burned, sleep duration, and activity intensity levels.

## 6.2 Current Health Analysis — K-Means Clustering

K-Means clustering was used to segment patients into health clusters based on their current-day readings. Each cluster corresponds to a risk level:

- Cluster A → Normal (healthy activity levels, stable vitals)

- Cluster B → Medium Risk (mild drop in one or more metrics)

- Cluster C → High Risk (critical drop or multiple metrics below threshold)

## 6.3 Deterioration Detection Model

A separate model was developed to analyze historical trends rather than just current values. This model:

- Looks at rolling averages of steps, sleep, workout duration, and heart rate over days/weeks

- Detects sustained downward trends that indicate gradual health decline

- Flags patients showing deterioration even if any single day's readings appear acceptable

- Particularly important for elderly patients where gradual decline is more dangerous than sudden spikes

## 6.4 Risk Notification Engine

The system automatically generates on-screen notifications whenever the risk classification changes for any patient. Supervisors do not need to manually check — alerts surface proactively based on model outputs.

# 7. Installation & Setup Guide

## 7.1 Prerequisites — Install These First

Before doing anything, make sure the following three tools are installed on your machine:

| Tool | Version Required | Download Link |
| --- | --- | --- |
| Node.js | v18 or higher | https://nodejs.org |
| Python | v3.14+ | https://python.org/downloads |
| VS Code | Latest | https://code.visualstudio.com |

## 7.2 Step 1 — PowerShell Execution Policy (Windows Only)

Before running any scripts on Windows, you need to allow PowerShell to run local scripts. Open PowerShell as normal user and run:

Set-ExecutionPolicy RemoteSigned -Scope CurrentUser # When prompted, press Y then Enter

## 7.3 Step 2 — Frontend Setup

Open the project folder in VS Code, then open the integrated terminal (Ctrl + `) and run:

# Navigate to the frontend folder cd health-path-monitor-main # Install all frontend dependencies npm install

✅  Frontend setup complete after npm install finishes without errors.

## 7.4 Step 3 — Backend Setup

Now navigate to the backend folder and install Python dependencies:

# Navigate into backend folder cd backend # Install all Python requirements pip install -r requirements.txt

⚠️  If you face errors with scikit-learn installation:

Some machines have issues compiling scikit-learn from source. Run this command to force a pre-built binary install, then re-run requirements:

# Force pre-built binary for scikit-learn pip install scikit-learn --only-binary=:all: # Then re-run requirements pip install -r requirements.txt

## 7.5 Step 4 — Train the ML Models

The ML models need to be trained before the backend can classify risk levels. Run these commands from the backend folder:

# Make sure you are inside the backend folder cd backend # Train the K-Means current health model python train_health_model.py # Train the deterioration / trend detection model python train_trend_model.py

## 7.6 Step 5 — Start the Backend Server

Once models are trained, start the Flask/Python backend server:

python app.py # Backend will start running on http://localhost:5000

✅  Backend is now live. Keep this terminal open while using the app.

## 7.7 Step 6 — Start the Frontend

Open a second terminal in VS Code, navigate back to the frontend folder, and start the React app:

cd health-path-monitor-main npm start # App will open at http://localhost:3000

| # | Step | Command / Action |
| --- | --- | --- |
| 1 | Set PowerShell policy | Set-ExecutionPolicy RemoteSigned -Scope CurrentUser → Y |
| 2 | Go to frontend folder | cd health-path-monitor-main |
| 3 | Install frontend deps | npm install |
| 4 | Go to backend folder | cd backend |
| 5 | Install Python deps | pip install -r requirements.txt |
| 6 | Fix scikit-learn (if error) | pip install scikit-learn --only-binary=:all: |
| 7 | Train health model | python train_health_model.py |
| 8 | Train trend model | python train_trend_model.py |
| 9 | Start backend | python app.py |
| 10 | Start frontend | cd health-path-monitor-main → npm start |

## 7.8 Android App (Health Connect Sync) Setup

- Open the Health Connect Sync project in Android Studio

- Add your google-services.json from Firebase to the app/ directory

- Ensure Health Connect is installed on the target device (Android 9+)

- Grant all required permissions: Steps, Heart Rate, Sleep, Activity

- Build and install the APK on the patient's Android phone

- The app will automatically sync data to Firebase on a scheduled interval

## 7.9 Watch & Zepp Configuration

- Install the Zepp app from the Google Play Store

- Pair the Amazefit Pace with Zepp via Bluetooth

- In Zepp settings, enable Health Connect data sharing

- Verify sync is active: Zepp → Settings → Third-party platforms → Health Connect

# 8. Firebase Database Structure

Data is organized in Firebase Realtime Database with the following hierarchy:

healthsync-db/ patients/ P1/ profile/ { name, age, assignedDoctor, assignedSupervisor } healthData/ [timestamp]/ { heartRate, steps, calories, distance, sleep, workout } riskLevel/ { current, lastUpdated } users/ [uid]/ { role, name, assignedPatients[] }

# 9. Future Enhancements

- Add support for blood pressure (BP) monitors via Bluetooth LE integration

- Expand wearable support to other brands (Fitbit, Garmin, Apple Watch via HealthKit)

- Build a mobile supervisor app (iOS/Android) for on-the-go monitoring

- Add emergency SOS alert — auto-call nurse station if critical threshold breached

- Integrate video/audio note-taking for doctors within patient profiles

- Multi-facility support: one admin can manage multiple care home branches

- Add PDF export of weekly health reports per patient

- Implement LSTM/time-series deep learning for more accurate deterioration forecasting

# 10. Project Credits

| Project Name | HealthSync — Personal Health Monitoring System |
| --- | --- |
| Domain | Healthcare Technology / IoT / Machine Learning |
| Database | Firebase Realtime Database |
| ML Dataset | Fitbit Fitness Tracker Dataset (Public) |
| Wearable Device | Amazefit Pace (Xiaomi) |
| Target Users | Old Age Homes, Hospitals, Care Facilities |
| System Type | Multi-patient, Multi-supervisor, Multi-doctor |

HealthSync — Keeping Every Heartbeat Counted Built with care for those who care.